BLE Beacon

Power Consumption Overview and Optimization

Andreas Erdmann

[Kurstitel]

Inhalt

2	Baseline	10

3	The baseline for the device is measured with only the Softdevice and no app flashed.	10

4	According to Specification	10

5	Nrf52 according to spec TDB	10

6	Device KX022 high power mode   145 uA@2.5V low power mode  10   uA standby   0.9  uA	10

7	Device SHT3   idle state    0.2 uA (max 2 uA) Average   2 uA while measuring w/lowest repeat.+single shot)	10

8	Case 1: Softdevice flashed, no app flashed	11

9		11

10	Case 2: Softdevice, just go to idle mode	11

11		11

12	Case 3: Softdevice, init bsp (led off), and just go to idle mode	11

13		11

14	Case 3b: as case 3 w/one led on	11

15		12

16	Case 4: as case 3 (led off) + twi_config + (both) sensor_init	12

17		12

18	Case 4a: as case 4 but no sensor_init	12

19		12

20	Case 4b: only kx022 init	13

21	Case 4c: only SHT3 init 	13

22	Case 5: as case 4c/SHT but no KX022, no sensor data + BLE adv 	13

23	Case 6: as case 5+ SAADC measurement)	14

24	Case 6a: as case 6 w/o BLE init/adv 	14

25	Case 7: as case 5 + SHT measurement	14

26	Case 8: all but KX022 measurement, 1/8 data acquisition for SHT and SAADC 	15

27	Case 8a	15

28	reduce transmit power  to 0dBm  from +4dBm	15

29	adv int    to 1 sec from 1/10 sec	15

30	SHT update int  to 5 sec from 1/8 sec	15

31	SADC update int  to 10 sec from 1/8 sec	15

32		15

33	Sensor update (all 5 sec)	15

34	Adv (all 1 sec)	16

35	Startup	16

36	Peaks	16

36.1	Power on peak	16

36.2	First adv	16

36.3	First sensor acquisition	16

37	Part 2 – Power Optimization KX022 Accelerometer	16

38	Baseline, no BLE, no sensor init	16

39	Change to TWI without transaction manager, SHT3 init and KX022 init to standby	17

40	With KX022 and SHT3 “one shot” measurement, 1 Hz	17

41	One measurement 	18

42	Init KX022, Standby    1,2ms wait 1.2/ODR      3ms set to operate     0,5ms wait 1.2/ODR for value    3ms read accel values     … SUM      ~8ms	18

43	Set SHT3 to SHT3_MEAS_HIGHREP_STRETCH wait clock stretch     12,5ms read temperature and humidity   2,8ms SUM      ~15ms	18

44	Process data and sleep again… Overall cycle     25ms, avg. power consumption 3,5mA, idle < 4uA	18

45	Overall 	18

46	One 10sec cycle 	19

47	cycle 	19

48	BLE 0 dBm, adv. interval   1s sensor (SHT3 and KX022) interval  5s SAADC (battery level) interval  10s overall power consumption   ~30uA (28,77uA) idle power consumption  3,5uA	19

49	Part 3 – Use RTC INT for while waiting for accel data	19

50	Baseline 	19

51	Use RTC counter (freq 1/256) for KX022 “put to operation”, “wait for accel data”, and during SHT3 temp/hum measurement (w/max. 15ms time) 	20

51.1	20

52	one minute, analog to Part 2 overall 	20

53	Optimization summary (Part 3  Part 4) BLE 0 dBm, adv. interval   1s sensor (SHT3 and KX022) interval  5s SAADC (battery level) interval  10s overall power consumption   ~30uA (28,77uA)   17,68uA  idle power consumption  3,5uA    3,5 uA	20

54	Part 4 – Further optimization	20

55	Using nested approach: start long running SHT3 first, complete KX022 tasks and read SHT3 values KX022: ODR 1600 -> delay time 3ms 	21

56	one minute, analog to Part 3 overall 	21

57	thus, no real further improvement	21

58	KX022: ODR 200 -> delay time 7ms 	21

59	one minute, analog to Part 3 overall 	22

60	thus, no real further improvement	22

62	Longer intervals between adv and samples	23

63	BLE 0 dBm, adv. interval   1s sensor (SHT3 and KX022) interval  15s SAADC (battery level) interval  60s overall power consumption   14,4uA  idle power consumption  3,6 uA	23

64	(> 1 Jahr = 365*24h = 8760h; CR2032 = 220 mAh)	23

65	Button functionality introduced	24

66		24

67	Single button press 	24

68	Original Beacon Firmware (for comparison)	24

69		24

70	Frequent spikes (5 Hz)	24

71	Larger but rarer spikes (1 Hz)	25

# Baseline

The baseline for the device is measured with only the Softdevice and no app flashed.

## According to Specification

Nrf52 according to spec
TDB

Device KX022

high power mode 		145 uA@2.5V

low power mode		10   uA

standby			0.9  uA

Device SHT3

idle state 			0.2 uA (max 2 uA)

Average			2 uA while measuring w/lowest repeat.+single shot)

## Initial Measuring

### Softdevice flashed, no app flashed

Figure 1 Softdevice flashed, no app flashed

Softdevice, just go to idle mode

# Figure 2 Softdevice, just go to idle mode

# Case 3: Softdevice, init bsp (led off), and just go to idle mode

# Case 3b: as case 3 w/one led on

# Case 4: as case 3 (led off) + twi_config + (both) sensor_init

# Case 4a: as case 4 but no sensor_init

# Case 4b: only kx022 init

# Case 4c: only SHT3 init

# Case 5: as case 4c/SHT but no KX022, no sensor data + BLE adv

# Case 6: as case 5+ SAADC measurement)

# Case 6a: as case 6 w/o BLE init/adv

# Case 7: as case 5 + SHT measurement

# Case 8: all but KX022 measurement, 1/8 data acquisition for SHT and SAADC

# Case 8a

# reduce transmit power 	to 0dBm 	from +4dBm

# adv int 			to 1 sec	from 1/10 sec

# SHT update int		to 5 sec	from 1/8 sec

# SADC update int		to 10 sec	from 1/8 sec

# Sensor update (all 5 sec)

# Adv (all 1 sec)

# Startup

# Peaks

## Power on peak

## First adv

## First sensor acquisition

# Part 2 – Power Optimization KX022 Accelerometer

# Baseline, no BLE, no sensor init

# Change to TWI without transaction manager, SHT3 init and KX022 init to standby

# With KX022 and SHT3 “one shot” measurement, 1 Hz

# One measurement

# Init KX022, Standby				1,2ms
wait 1.2/ODR 					3ms
set to operate					0,5ms
wait 1.2/ODR for value				3ms
read accel values					…
SUM						~8ms

# Set SHT3 to SHT3_MEAS_HIGHREP_STRETCH
wait clock stretch					12,5ms
read temperature and humidity			2,8ms
SUM						~15ms

# Process data and sleep again…
Overall cycle					25ms, avg. power consumption 3,5mA, idle < 4uA

# Overall

# One 10sec cycle

# cycle

# BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	5s
SAADC (battery level) interval 	10s
overall power consumption 		~30uA (28,77uA)
idle power consumption		3,5uA

# Part 3 – Use RTC INT for while waiting for accel data

# Baseline

# Use RTC counter (freq 1/256) for KX022 “put to operation”, “wait for accel data”, and during SHT3 temp/hum measurement (w/max. 15ms time)

# one minute, analog to Part 2 overall

# Optimization summary (Part 3  Part 4)
BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	5s
SAADC (battery level) interval 	10s
overall power consumption 		~30uA (28,77uA)		 17,68uA 
idle power consumption		3,5uA			 3,5 uA

# Part 4 – Further optimization

# Using nested approach: start long running SHT3 first, complete KX022 tasks and read SHT3 values
KX022: ODR 1600 -> delay time 3ms

# one minute, analog to Part 3 overall

# thus, no real further improvement

# KX022: ODR 200 -> delay time 7ms

# one minute, analog to Part 3 overall

# thus, no real further improvement

# Longer intervals between adv and samples

# BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	15s
SAADC (battery level) interval 	60s
overall power consumption 		14,4uA 
idle power consumption		3,6 uA

#### 220mAh / 0,0144mA * 0,7 = 10.694 h = ~1.2 Jahre

# (> 1 Jahr = 365*24h = 8760h; CR2032 = 220 mAh)

# Button functionality introduced

# Single button press

# Original Beacon Firmware (for comparison)

# Frequent spikes (5 Hz)

# Larger but rarer spikes (1 Hz)