Case 1: Softdevice flashed, no app flashed

per data sheet

KX022	high power mode 	145 uA	@2.5V

low power mode	10   uA

standby		0.9  uA

SHT3	idle state 		0.2 uA		(max 2 uA)

Average		2     uA 		while measuring w/lowest repeat.+single shot)

Case 2: Softdevice, just go to idle mode

Case 3: Softdevice, init bsp (led off), and just go to idle mode

Case 3b (one led on)

Case 4: (led off=3 + twi_config + (both) sensor_init)

Case 4a: = 4 but no sensor_init

Case 4b: only kx022 init

Case 4c: only SHT3 init

Case 5 (case 4c/SHT but no KX022, no sensor data + BLE adv)

Case 6 (case5+ saadc measurement)

Case 6a (case 6 w/o BLE init/adv)

Case 7 (case 5 + SHT measurement)

Case 8 (all but KX022 measurement, 1/8 data acquisition for SHT and SAADC)

Case 8a

- reduce transmit power 	to 0dBm 	from +4dBm

- adv int 			to 1 sec	from 1/10 sec

- SHT update int		to 5 sec	from 1/8 sec

- SADC update int		to 10 sec	from 1/8 sec

Sensor update (all 5 sec)

Adv (all 1 sec)

Startup

- Peaks

  - Power on peak

  - First adv

  - First sensor acquisition

# Part 2 – Power Optimization KX022 Accelerometer

Baseline, no BLE, no sensor init

Change to TWI without transaction manager, SHT3 init and KX022 init to standby

With KX022 and SHT3 “one shot” measurement, 1 Hz

One measurement

Init KX022, Standby				1,2ms
wait 1.2/ODR 					3ms
set to operate					0,5ms
wait 1.2/ODR for value				3ms
read accel values					…
SUM						~8ms

Set SHT3 to SHT3_MEAS_HIGHREP_STRETCH
wait clock stretch					12,5ms
read temperature and humidity			2,8ms
SUM						~15ms

Process data and sleep again…
Overall cycle					25ms, avg. power consumption 3,5mA, idle < 4uA

# Part 3 – Overall

One 10sec cycle

Startup cycle

BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	5s
SAADC (battery level) interval 	10s
overall power consumption 		~30uA (28,77uA)
idle power consumption		3,5uA