# Part 1 – Baseline

The baseline for the device is measured with only the Softdevice and no app flashed.

## Calculation

Nrf52 calculation:

- TDB

Device KX022

- high power mode 		145 uA@2.5V

- low power mode		10   uA

- standby			0.9  uA

Device SHT3

- idle state 			0.2 uA (max 2 uA)

- Average			2 uA while measuring w/lowest repeat.+single shot)

## Case 1: Softdevice flashed, no app flashed

## Case 2: Softdevice, just go to idle mode

## Case 3: Softdevice, init bsp (led off), and just go to idle mode

## Case 3b: as case 3 w/one led on

## Case 4: as case 3 (led off) + twi_config + (both) sensor_init

## Case 4a: as case 4 but no sensor_init

## Case 4b: only kx022 init

## Case 4c: only SHT3 init

## Case 5: as case 4c/SHT but no KX022, no sensor data + BLE adv

## Case 6: as case 5+ SAADC measurement)

## Case 6a: as case 6 w/o BLE init/adv

## Case 7: as case 5 + SHT measurement

## Case 8: all but KX022 measurement, 1/8 data acquisition for SHT and SAADC

## Case 8a

- reduce transmit power 	to 0dBm 	from +4dBm

- adv int 			to 1 sec	from 1/10 sec

- SHT update int		to 5 sec	from 1/8 sec

- SADC update int		to 10 sec	from 1/8 sec

### Sensor update (all 5 sec)

### Adv (all 1 sec)

### Startup

- Peaks

  - Power on peak

  - First adv

  - First sensor acquisition

# Part 2 – Power Optimization KX022 Accelerometer

## Baseline, no BLE, no sensor init

## Change to TWI without transaction manager, SHT3 init and KX022 init to standby

## With KX022 and SHT3 “one shot” measurement, 1 Hz

### One measurement

Init KX022, Standby				1,2ms
wait 1.2/ODR 					3ms
set to operate					0,5ms
wait 1.2/ODR for value				3ms
read accel values					…
SUM						~8ms

Set SHT3 to SHT3_MEAS_HIGHREP_STRETCH
wait clock stretch					12,5ms
read temperature and humidity			2,8ms
SUM						~15ms

Process data and sleep again…
Overall cycle					25ms, avg. power consumption 3,5mA, idle < 4uA

## Overall

### One 10sec cycle

### cycle

BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	5s
SAADC (battery level) interval 	10s
overall power consumption 		~30uA (28,77uA)
idle power consumption		3,5uA

# Part 3 – Use RTC INT for while waiting for accel data

## Baseline

## Use RTC counter (freq 1/256) for KX022 “put to operation”, “wait for accel data”, and during SHT3 temp/hum measurement (w/max. 15ms time)

## one minute, analog to Part 2 overall

Optimization summary (Part 3  Part 4)
BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	5s
SAADC (battery level) interval 	10s
overall power consumption 		~30uA (28,77uA)		 17,68uA 
idle power consumption		3,5uA			 3,5 uA

# Part 4 – Further optimization

### Using nested approach: start long running SHT3 first, complete KX022 tasks and read SHT3 values
KX022: ODR 1600 -> delay time 3ms

### one minute, analog to Part 3 overall

thus, no real further improvement

### KX022: ODR 200 -> delay time 7ms

### one minute, analog to Part 3 overall

thus, no real further improvement

## Longer intervals between adv and samples

BLE 0 dBm, adv. interval 		1s
sensor (SHT3 and KX022) interval 	15s
SAADC (battery level) interval 	60s
overall power consumption 		14,4uA 
idle power consumption		3,6 uA

220mAh / 0,0144mA * 0,7 = 10.694 h = ~1.2 Jahre

(> 1 Jahr = 365*24h = 8760h; CR2032 = 220 mAh)

Button functionality introduced

Single button press